# 4EB00 Jet Engine Project

Working plan, technical roadmap and AI handoff
Group 10

Purpose of this document This is a working guide for the team. It explains what each project part must do, which files and equations are needed, what information must be passed to the next part, and how to check the result. It is also written so that a teammate or an AI assistant can receive this file later and continue the project step by step without guessing missing assumptions.

Engine chain

1  →  Diffuser  →  2  →  Compressor  →  3  →  Combustor  →  4  →  Turbine  →  5  →  Nozzle  →  6

| Project split | Main responsibility | Output to next person |
| --- | --- | --- |
| Part 1 | Setup + diffuser + compressor (states 1–3) | State 3 + compressor power requirement |
| Part 2 | Combustion chemistry + combustor (state 3–4) | State 4 + product composition + product mass flow |
| Part 3 | Turbine + nozzle (states 4–6) | States 5–6 + exhaust velocity |
| Part 4 | Integration + validation + results + report | One working model + checked report/submission |

Working document - update names, assumptions and exact efficiency values once they are confirmed from the current Canvas/template files.

# 0. Read this first

Non-negotiable modeling rules Use the NASA thermodynamic database for temperature-dependent properties. Do not replace the requested method with Poisson relations, a constant-cp model, or an LHV heat-addition shortcut. Do not invent missing efficiencies, pressure losses or inlet temperatures. If an assumption is not in the current files, locate it in the template/book/Canvas material before finalizing the code.

☐ Use Group 10 inputs only. Do not copy sample-group values from the lecture slides.

☐ Keep all internal units consistent: T in K, P in Pa, h in J/kg, s in J/(kg K), mass flow in kg/s, velocity in m/s.

☐ Do not hard-code intermediate answers such as T2, T3, T4, P5 or v6. Every downstream value must be calculated from previous states.

☐ Use the same species order everywhere. Recommended order for Group 10: [H2, O2, CO2, H2O, N2].

☐ Before combustion use air properties. After combustion use product-mixture properties.

☐ Every part must include physical and numerical checks, not only calculations.

☐ Before submission, run the final package from a clean folder to make sure relative paths work.

## Group 10 inputs

| Input | Value | Meaning |
| --- | --- | --- |
| Fuel | H2 | Hydrogen fuel |
| Tamb | 300 K | Ambient / state-1 temperature |
| Pamb | 100000 Pa | Ambient / state-1 pressure |
| P3/P2 | 9 | Compressor pressure ratio |
| Fuel mass flow | 0.58 kg/s | m_dot_f |
| AF | 204.42 | Air-to-fuel mass ratio |
| v1 | 200 m/s | Flight / inlet velocity |

m_dot_air = AF · m_dot_f = 204.42 · 0.58 = 118.5636 kg/s

m_dot_products = m_dot_air + m_dot_f = 119.1436 kg/s

Important open items - do not guess The exact compressor/turbine/nozzle isentropic efficiencies, fuel inlet state/temperature, combustor pressure-loss assumption, shaft mechanical-loss assumption and exact nozzle-efficiency definition must be taken from the current assignment template/book/Canvas material. The lecture states that component efficiencies are given, but the numerical values were not present in the material summarized here.

# 1. Source map - where to find the information

| File / folder | Use it for | What to look for |
| --- | --- | --- |
| 4EB00 Special Topic: The Jet Engine Info 2025.pdf | Official assignment scope and deliverables | NASA-database approach; MATLAB requirement; working script + compact report; Turns references. Treat 2025 dates/group-size statements as potentially outdated for 2026. |
| Lecture 1 - Ideal Gas Mixtures 2024.pdf | Mixtures, reacting systems, combustion | Mole/mass fractions; mixture properties; formation enthalpy; NASA-polynomial use; combustion/product composition. |
| Lecture 2 - Cycle Analysis 2025.pdf | Full engine control-volume method | States 1–6; diffuser example; compressor ratio; T4 computed by model; P6=Pamb; component efficiencies; NASA entropy method. |
| Starter / template MATLAB script | Base code to extend | Keep its setup and database-loading logic; replace sample group values with Group 10 values. |
| General/NasaThermalDatabase.mat | Species property database | Temperature-dependent NASA coefficients. |
| General/HNasa.m | Specific enthalpy | h_i(T), used heavily in all parts. |
| General/SNasa.m | Temperature part of entropy | Used for isentropic reference states and pressure relations. |
| General/CpNasa.m and CvNasa.m | Heat capacities | Mainly for understanding/checks; not a replacement for NASA enthalpy/entropy calculations. |
| General/UNasa.m | Specific internal energy | Useful for closed/reacting-system examples; less central in the steady-flow engine model. |
| General/myfind.m | Species selection | Find H2, O2, CO2, H2O, N2 in database. |
| GroupSettings.zip / current Canvas group file | Exact group data | Confirm Group 10 values and any group-specific settings not already listed. |
| Current Canvas report Word template | Report structure | Follow strictly. Do not invent a different final report structure. |
| Turns textbook: Jet Engine / Turbojet section | Component definitions and exact model details | 1st ed. pp. 551–554 or 2nd ed. pp. 556–560 as referenced in the assignment; use the edition you have. |

AI / teammate handoff rule If any of the files above are missing, ask for that file before inventing an assumption. Use this document as the roadmap, but use the official template/book/Canvas material as the authority for numerical efficiencies and component-specific details.

# 2. Common MATLAB setup used by all four parts

The final model should be one sequential script. Part 1 establishes the common database/species framework, and Parts 2–4 reuse it rather than recreating independent models.

Recommended final script order SETUP → GROUP 10 INPUTS → NASA DATABASE / SPECIES → AIR PROPERTIES → DIFFUSER 1→2 → COMPRESSOR 2→3 → COMBUSTION / COMBUSTOR 3→4 → PRODUCT PROPERTIES → TURBINE 4→5 → NOZZLE 5→6 → FINAL RESULTS → CHECKS

Starter setup (keep the same logic as the supplied template):

clear all; close all; clc; warning off relativepath_to_generalfolder = 'General'; addpath(relativepath_to_generalfolder); TdataBase = fullfile('General','NasaThermalDatabase'); load(TdataBase); global Runiv Pref Runiv = 8.314472; Pref = 1.01235e5; Tref = 298.15;

## Species and air mixture

Species order: [H2, O2, CO2, H2O, N2]

X_air ≈ [0, 0.21, 0, 0, 0.79]

Convert mole fractions X_i to mass fractions Y_i using species molecular masses from the database. The property mixture should then be formed with mass fractions, for example:

h_mix(T) = Σ Y_i h_i(T)

s_T,mix(T) = Σ Y_i s_T,i(T)

A practical precomputation over a temperature range is useful:

TR = 200:1:3000; for i = 1:length(SpS) hia(:,i) = HNasa(TR,SpS(i)); sia(:,i) = SNasa(TR,SpS(i)); end hair_a = Yair * hia'; sair_a = Yair * sia';

Core numerical pattern used repeatedly Known property → evaluate a NASA mixture-property curve → invert the curve to find temperature. Example: known h2 → solve h_air(T2)=h2 with interpolation/bisection. The same idea appears again for T3s, T4, T5, T5s, T6s and T6.

# 3. Part 1 - Setup, diffuser and compressor (states 1–3)

| Owner | Status | Start input | End output |
| --- | --- | --- | --- |
| ________________ | ☐ Not started ☐ In progress ☐ Done | Group 10 ambient data + NASA setup | T3, P3, h3, s3 + compressor work requirement |

Part 1 goal Take ambient air at state 1, model the diffuser to state 2, then model the compressor to state 3. This section also establishes the common MATLAB/NASA framework that the rest of the project reuses.

## 3.1 State 1 - ambient inlet

T1 = Tamb = 300 K

P1 = Pamb = 100000 Pa

v1 = 200 m/s

Use the NASA air-mixture enthalpy and entropy functions to establish h1 and the thermal entropy term at T1. State 1 should end with T1, P1, v1, h1 and the entropy information needed for the diffuser.

## 3.2 Diffuser 1→2

Physical idea: the diffuser slows the incoming air. Kinetic energy is converted into enthalpy, so temperature and pressure normally rise.

h1 + v1²/2 = h2 + v2²/2

Under the starter-model assumption v2 ≈ 0:

h2 = h1 + v1²/2

Then solve the NASA air enthalpy curve for T2:

h_air(T2) = h2

The diffuser is treated as isentropic:

s2 = s1

s_T(T2) - s_T(T1) - R_air ln(P2/P1) = 0

P2 = P1 exp[(s_T(T2)-s_T(T1))/R_air]

Expected Group 10 scale from the earlier check: roughly T2 ≈ 320 K and P2 ≈ 125 kPa. These are only sanity-check values; do not hard-code them.

☐ Energy residual for diffuser is near zero.

☐ T2 > T1.

☐ P2 > P1.

☐ The value h_air(T2) reproduces the calculated h2 within interpolation tolerance.

## 3.3 Compressor 2→3

P3 = 9 P2

First find the ideal isentropic reference state 3s:

s3s = s2

s_T,3s = s_T,2 + R_air ln(P3/P2)

Invert the NASA air entropy curve to find T3s, then obtain h3s = h_air(T3s). For the actual compressor use the specified compressor isentropic efficiency from the official template/book:

η_c = (h3s - h2)/(h3 - h2)

h3 = h2 + (h3s - h2)/η_c

Finally invert h_air(T3)=h3 to get T3. Compute the compressor power requirement (magnitude):

W_dot_c,in = m_dot_air (h3 - h2)

Do not finalize this section until η_c is located The lecture says component isentropic efficiencies are given, but the numerical value is not in this working document. Find the exact compressor efficiency in the current template/book/Canvas material.

## 3.4 Part 1 handoff

| Quantity | Pass to | Why it is needed |
| --- | --- | --- |
| T3, P3, h3, s3 | Part 2 | Combustor inlet state |
| m_dot_air | Part 2 and Part 3 | Combustor mass balance and compressor/turbine shaft balance |
| h2, h3 or W_dot_c,in | Part 3 | Turbine must drive the compressor |
| Y_air + NASA arrays | Part 2 | Reactant mixture and common property infrastructure |

Part 1 definition of done The code runs from the start of the script through state 3 without manual intermediate inputs; the diffuser and compressor checks pass; and the outputs above are ready for direct use by Parts 2 and 3.

# 4. Part 2 - Combustion chemistry and combustor (state 3→4)

| Owner | Status | Start input | End output |
| --- | --- | --- | --- |
| ________________ | ☐ Not started ☐ In progress ☐ Done | State 3 + H2 fuel + AF | T4, P4, h4, s4, Y_products, m_dot_products |

Part 2 goal Determine what the gas becomes when H2 burns in the compressed air, then use a steady-flow energy balance with NASA enthalpies to calculate the combustor exit temperature T4. This is the core reacting-system section of the project.

## 4.1 Mass flows

m_dot_air = AF · m_dot_f = 118.5636 kg/s

m_dot_products = m_dot_air + m_dot_f = 119.1436 kg/s

Part 2 receives the exact state-3 values from Part 1. Never type a copied T3 or h3 value manually into this section.

## 4.2 Reaction and product composition

H2 + 1/2 O2 → H2O

Nitrogen passes through in the simplified model. Because the fuel contains no carbon, CO2 remains zero for this H2 case. With the high air-to-fuel ratio there should be excess oxygen in the products.

☐ Use species molecular masses from SpS/database, not memorized rounded values if avoidable.

☐ Calculate O2 available in incoming air.

☐ Calculate O2 required by 0.58 kg/s H2.

☐ Calculate remaining O2 after reaction.

☐ Calculate H2O formed.

☐ Carry N2 through unchanged.

☐ Build product mole fractions X_products and mass fractions Y_products in the agreed species order.

☐ Verify ΣX_products = 1 and ΣY_products = 1.

☐ Verify total product mass flow equals m_dot_air + m_dot_f.

☐ Verify elemental H, O and N balances.

The product mass-fraction vector should have the form:

Y_products = [0, Y_O2, 0, Y_H2O, Y_N2]

Earlier rough calculations gave approximately [0, 0.193, 0, 0.0435, 0.7635]. Use this only as a sanity check; calculate the final vector from database molecular masses.

## 4.3 Build product NASA properties

h_products(T) = Σ Y_i,products h_i(T)

s_T,products(T) = Σ Y_i,products s_T,i(T)

In MATLAB this is conceptually the same as the air mixture, but with Yprod instead of Yair. From state 4 onward, the turbine and nozzle must use the product-mixture curves.

## 4.4 Combustor energy balance and T4

For a steady combustor with no shaft work and under the heat-loss/KE/PE assumptions specified by the official model, the central balance is:

m_dot_air h_air,3 + m_dot_f h_fuel,in = m_dot_products h_products,4

Therefore the target outlet specific enthalpy is:

h4,target = [m_dot_air h3 + m_dot_f h_fuel,in] / m_dot_products

Then solve the product enthalpy curve for temperature:

h_products(T4) = h4,target

Why there is no extra LHV term NASA species enthalpies include the chemical formation-enthalpy contribution. The chemical energy difference between reactants and products is therefore already present in the enthalpy balance. Adding a separate fuel LHV on top of those full NASA enthalpies can double-count chemical energy.

Still needs confirmation from official files Before finalizing Part 2, confirm the fuel inlet temperature/state and the combustor pressure-loss assumption (for example whether P4=P3 is assumed or a pressure loss is specified). Do not guess either one.

## 4.5 Part 2 checks and handoff

☐ m_dot_products = m_dot_air + m_dot_f.

☐ Elemental H/O/N balances close.

☐ ΣX_products = 1 and ΣY_products = 1.

☐ Combustor energy residual is near zero.

☐ T4 > T3 by a physically sensible amount.

☐ State-4 entropy information is evaluated using product properties, not air properties.

| Quantity | Pass to Part 3 | Reason |
| --- | --- | --- |
| T4, P4, h4, s4 | Yes | Turbine inlet state |
| Y_products | Yes | Turbine/nozzle thermodynamic properties |
| m_dot_products | Yes | Turbine and nozzle mass flow |
| Product h(T), s_T(T) arrays or equivalent calculation | Yes | Needed for states 5, 5s, 6 and 6s |

Part 2 definition of done The reaction composition is generated from first principles, all mass/element checks pass, the NASA product-mixture property curves are available, and T4 is solved from the combustor energy balance without an LHV shortcut.

# 5. Part 3 - Turbine and nozzle (states 4–6)

| Owner | Status | Start input | End output |
| --- | --- | --- | --- |
| ________________ | ☐ Not started ☐ In progress ☐ Done | State 4 + product mixture + compressor requirement | State 5, state 6, turbine power, exhaust velocity v6 |

Part 3 goal Use the turbine to extract enough work from the hot combustion products to drive the compressor, then use the nozzle to convert the remaining enthalpy into exhaust kinetic energy. The main end result is the complete state 6 and the exhaust velocity.

## 5.1 Turbine 4→5 - shaft balance

The compressor and turbine are connected by a shaft. Under the no-shaft-loss assumption (only if that matches the official model):

W_dot_t = W_dot_c

m_dot_products (h4 - h5) = m_dot_air (h3 - h2)

h5 = h4 - [m_dot_air/m_dot_products] (h3 - h2)

Then solve h_products(T5)=h5 to find the actual turbine exit temperature T5.

Check the shaft assumption If the template specifies a mechanical shaft efficiency or accessory load, include it. Do not silently assume perfect shaft transmission if the official model says otherwise.

## 5.2 Turbine isentropic reference state 5s

Use the exact turbine isentropic efficiency from the template/book:

η_t = (h4 - h5)/(h4 - h5s)

h5s = h4 - (h4 - h5)/η_t

Solve h_products(T5s)=h5s. For the ideal reference expansion:

s5s = s4

P5 = P4 exp[(s_T,5s - s_T,4)/R_products]

After P5 is known, evaluate the actual entropy s5 using T5, P5 and the product mixture.

☐ T5s < T5 < T4 for a nonideal turbine.

☐ P5 < P4.

☐ For an adiabatic irreversible turbine, actual s5 should be greater than s4.

☐ The turbine shaft power matches the required compressor power under the chosen shaft model.

## 5.3 Nozzle 5→6

The lecture specifies the nozzle exit pressure:

P6 = Pamb = 100000 Pa

First calculate an ideal isentropic reference state 6s:

s6s = s5

s_T,6s = s_T,5 + R_products ln(P6/P5)

Invert the product entropy curve for T6s, then calculate h6s = h_products(T6s).

For the ideal nozzle energy balance:

h5 + v5²/2 = h6s + v6s²/2

v6s = √[v5² + 2(h5 - h6s)]

If the model neglects nozzle inlet velocity, use v5≈0. Use the exact nozzle-efficiency definition from the template/book to obtain the actual state 6. A common definition is:

η_n = (h5 - h6)/(h5 - h6s) = (v6² - v5²)/(v6s² - v5²)

h6 = h5 - η_n (h5 - h6s)

v6 = √[v5² + 2(h5 - h6)]

Unit warning for the nozzle The kinetic-energy equations require enthalpy differences in J/kg. If a value has been converted to kJ/kg for display, convert it back before taking the square root.

☐ P6 = Pamb exactly as required by the model.

☐ T6 < T5 and h6 < h5.

☐ v6 is much larger than the nozzle inlet velocity.

☐ For η_n < 1, v6 < v6s.

☐ Nozzle energy residual is near zero.

## 5.4 Part 3 handoff

| Output | Part 4 uses it for |
| --- | --- |
| T5, P5, h5, s5 | Complete state table and turbine checks |
| T6, P6, h6, s6 | Complete state table |
| v6 | Final engine result / any required thrust or performance calculation |
| W_dot_t | Shaft-balance validation |
| Optional ideal states 5s and 6s | Efficiency/entropy validation |

Part 3 definition of done The turbine is tied correctly to the compressor requirement, all turbine states are calculated with product-mixture NASA properties, the nozzle expands to ambient pressure, and the final exhaust velocity is obtained from the nozzle energy balance.

# 6. Part 4 - Integration, validation, results and report

| Owner | Status | Start input | End output |
| --- | --- | --- | --- |
| ________________ | ☐ Not started ☐ In progress ☐ Done | Parts 1–3 code and outputs | One working model + validated results + report/submission package |

Part 4 goal Turn the three technical sections into one defensible project. This person is not only the report writer: they are the integrator and checker who makes sure variables connect correctly, units and mixtures stay consistent, every conservation law closes, and the final package runs from a clean folder.

## 6.1 Integrate one sequential script

☐ Group 10 inputs appear once near the top of the script.

☐ Part 2 directly uses Part 1 outputs; Part 3 directly uses Parts 1 and 2 outputs.

☐ No copied intermediate numerical answers are re-entered manually.

☐ All paths are relative (for example, General folder relative to the main script).

☐ Air property arrays are used only before combustion; product property arrays are used from state 4 onward.

☐ All variables have consistent names and units.

## 6.2 Build the final state table

| State | Location | Mixture | Main values to report |
| --- | --- | --- | --- |
| 1 | Engine inlet | Air | T1, P1, h1, s1, v1 |
| 2 | Diffuser outlet | Air | T2, P2, h2, s2, v2 |
| 3 | Compressor outlet | Air | T3, P3, h3, s3 |
| 4 | Combustor outlet | Products | T4, P4, h4, s4 |
| 5 | Turbine outlet | Products | T5, P5, h5, s5 |
| 6 | Nozzle outlet | Products | T6, P6, h6, s6, v6 |

## 6.3 Global physical checks

| Component | Expected trend / condition |
| --- | --- |
| Diffuser | v decreases; T2>T1 and normally P2>P1 |
| Compressor | P3>P2 and T3>T2 |
| Combustor | T4>T3; composition changes from air to products |
| Turbine | T5<T4 and P5<P4 |
| Nozzle | P6=Pamb; h6<h5; exhaust velocity rises strongly |

## 6.4 Conservation-law residuals

Part 4 should calculate or at least explicitly verify residuals. A script that runs without errors is not enough.

Diffuser:  [h1 + v1²/2] - [h2 + v2²/2] ≈ 0

Combustor:  [m_dot_air h3 + m_dot_f h_fuel] - [m_dot_products h4] ≈ 0

Shaft:  W_dot_t - W_dot_c ≈ 0  (for the chosen shaft model)

Nozzle:  [h5 + v5²/2] - [h6 + v6²/2] ≈ 0

☐ Mass balance closes before and after fuel addition.

☐ Product mole and mass fractions sum to 1.

☐ H/O/N elemental balances close.

☐ Isentropic reference states satisfy s3s=s2, s5s=s4 and s6s=s5.

☐ All component efficiencies give physically correct actual-vs-ideal trends.

☐ No unexplained negative temperatures, pressures, mass fractions or imaginary nozzle velocities.

## 6.5 Final MATLAB output

At the end of the script, print a compact set of results and display the state table. A grader/teammate should not have to search the workspace for the important numbers.

| Result block | Recommended items |
| --- | --- |
| States | T, P, h, s for states 1–6; v where relevant |
| Mass flows | m_dot_air, m_dot_f, m_dot_products |
| Mixture | Final Y_products (and X_products if useful) |
| Power | Compressor power and turbine power |
| Nozzle | P6 and v6; ideal v6s if used as a check |
| Residuals | Key energy/mass/shaft residuals or pass/fail messages |

## 6.6 Report preparation

The report must follow the current Canvas Word template. Keep it compact: explain the thermodynamic model and assumptions, give the essential equations, then present the checked results. Do not paste long MATLAB code listings into the report unless the template specifically asks for them.

☐ Given Group 10 conditions are stated correctly.

☐ Diffuser/compressor model is explained using conservation laws + NASA properties.

☐ H2 reaction and product-composition method are explained.

☐ Combustor energy balance and how T4 was solved are explained.

☐ Turbine shaft connection and isentropic reference state are explained.

☐ Nozzle expansion to Pamb and exhaust-velocity calculation are explained.

☐ Final state/results table is included.

☐ Important model checks are mentioned.

☐ Any required performance quantities from the official report template (e.g. thrust/efficiency) are added only after confirming the exact requested definition.

About thrust / extra performance quantities A turbojet analysis can continue from v6 to thrust, specific thrust, fuel consumption or efficiency. This working plan does not declare those as mandatory because the summarized assignment pages do not explicitly list which final performance quantities the current report template requires. Check the actual template/Turns instructions, then add exactly what is requested.

## 6.7 Final packaging test

☐ Place the main script, helper files required by the assignment, and the General folder in the intended submission structure.

☐ Open a fresh MATLAB session.

☐ Run the main script from the first line without manually preloading variables.

☐ Confirm the NASA database loads via relative paths.

☐ Confirm the final state table and outputs appear.

☐ Confirm there are no warnings/errors caused by missing files or absolute user-specific paths.

☐ Export the report to PDF using the current Canvas naming rules.

☐ Verify current 2026 deadline, group-size rule and file naming on Canvas rather than relying on the 2025 handout.

Part 4 definition of done A fresh user can unzip/open the project, run the model from the top, obtain the same states 1–6 and final results, and understand from the report why the calculations are thermodynamically correct.

# 7. Interfaces between the four parts

The project will integrate smoothly only if each person treats the output of their section as an explicit interface rather than as informal numbers shared in chat.

| From | To | Must provide |
| --- | --- | --- |
| Part 1 | Part 2 | T3, P3, h3, s3, m_dot_air, Y_air, shared NASA arrays/functions |
| Part 1 | Part 3 | h2, h3 and/or W_dot_c,in, plus m_dot_air |
| Part 2 | Part 3 | T4, P4, h4, s4, Y_products, m_dot_products, product property curves |
| Parts 1–3 | Part 4 | Code blocks + outputs + assumptions + checks; no manually copied intermediate constants |

Rule for changes If Part 1 changes a state-3 calculation, Parts 2 and 3 should update automatically when the final script is rerun. If Part 2 changes product composition, Parts 3 and 4 should update automatically. That is the standard for a real integrated model.

# 8. Step-by-step instructions for an AI assistant or new teammate

Use this section as a handoff prompt. It deliberately states the order of operations and what not to assume.

Read the current official assignment info, current Canvas report template, both special-topic lectures, the group settings, and the starter MATLAB script before changing the model.

Confirm Group 10 input values and confirm the exact component efficiencies / loss assumptions. If any required value is absent, ask for the missing official file instead of inventing it.

Verify the General folder and NASA functions are accessible. Do not edit the supplied NASA base functions unless the assignment explicitly asks for it.

Establish the species order [H2, O2, CO2, H2O, N2], obtain molecular masses from the database, and build the air mass fractions from Xair=[0,0.21,0,0,0.79].

Precompute or otherwise evaluate air h(T) and s_T(T) using HNasa/SNasa.

Solve state 1 and the diffuser 1→2 with the steady-flow energy equation plus s2=s1.

Use P3/P2=9 to solve the compressor ideal state 3s from entropy, then the actual state 3 from the exact η_c.

Compute m_dot_air=AF·m_dot_f and the compressor power requirement.

For the combustor, build the H2 reaction stoichiometry, product species amounts, X_products and Y_products. Check total mass and elemental balances.

Build product h(T) and s_T(T) from NASA species properties. Use full NASA enthalpies (including formation enthalpy); do not add a separate LHV term unless the official model explicitly defines a different enthalpy convention.

Use the combustor energy balance to solve T4. Apply the official pressure-loss assumption to obtain P4 and evaluate h4/s4 consistently.

Use the turbine/compressor shaft balance to obtain h5. Use the exact η_t and s5s=s4 to determine the turbine pressure/state.

Use P6=Pamb and s6s=s5 for the ideal nozzle reference state, then the exact nozzle efficiency definition to obtain h6/T6 and the energy balance to obtain v6.

Integrate all sections into one sequential script. Remove hard-coded intermediate results and verify SI units.

Calculate mass, element, component-energy, shaft and isentropic-state residuals. Investigate any residual that is not near numerical tolerance.

Populate the official report template with the model assumptions, core equations, state table, final results and checks. Add thrust/efficiency only if the current template/book explicitly asks for them.

Run the complete package from a clean folder before submission.

# 9. First team meeting - practical agenda

Meeting objective Leave the meeting with one shared folder/repository, one agreed MATLAB main script, confirmed official assumptions, named owners for Parts 1–4, and a clear first checkpoint. Do not spend the whole meeting trying to finish the engine.

☐ Confirm current 2026 team-size rule and deadline on Canvas (the uploaded 2025 handout says groups of 2, so verify before relying on a four-person split for the official submission).

☐ Confirm everyone has the same starter files and General folder.

☐ Run the supplied starter/diffuser code on at least one machine.

☐ Confirm Group 10 values and fuel H2.

☐ Locate and write down the exact η_c, η_t, η_n, fuel inlet state, combustor pressure-loss rule and shaft-loss rule.

☐ Agree on the species order and shared variable naming.

☐ Assign an owner to each of Parts 1–4.

☐ Agree that one main script is the source of truth; people may prototype in copies/branches, but integration must be continuous.

☐ Set the first checkpoint: Part 1 diffuser/compressor logic working; Part 2 chemistry/product composition working; Part 3 equations prepared with dummy interfaces only if state 4 is not ready; Part 4 state-table/check framework created.

| Part | Owner | First checkpoint deliverable | Status |
| --- | --- | --- | --- |
| 1 | ____________ | NASA setup + states 1–2 working; compressor assumptions confirmed | ☐ |
| 2 | ____________ | H2 stoichiometry + product composition + checks | ☐ |
| 3 | ____________ | Turbine/nozzle equation/code skeleton using agreed interfaces | ☐ |
| 4 | ____________ | Integrated main-script structure + state table + residual/check section | ☐ |

# 10. Common mistakes to catch early

| Mistake | Why it is wrong / how to catch it |
| --- | --- |
| Using Poisson relations as the main project method | The assignment specifically wants NASA temperature-dependent properties and conservation of energy/entropy. |
| Using Y_air after the combustor | States 4–6 are combustion products; using air properties gives wrong h, s, T and pressure relations. |
| Adding LHV on top of full NASA formation enthalpies | Can double-count chemical energy. |
| Forgetting fuel mass in turbine/nozzle mass flow | After combustion, m_dot_products = m_dot_air + m_dot_f. |
| Assuming turbine mass flow equals compressor air flow | They differ because fuel is added before the turbine. |
| Using kJ/kg inside nozzle velocity formula without conversion | Produces a factor √1000 error in velocity. |
| Typing T3/T4/T5 manually into later parts | Breaks integration; changes upstream will not propagate. |
| Guessing efficiencies or pressure loss | Must come from official current model/template/book. |
| Absolute Windows/macOS path to General folder | Will fail on another group member/grader machine. |
| Code runs, but no balance checks | A numerical bug can still produce plausible-looking values; residuals catch this. |

# 11. Final project checklist

☐ Current official assumptions and efficiencies confirmed.

☐ Group 10 values used; sample-group values removed.

☐ NASA database loads from relative path.

☐ States 1–6 all calculated in one run.

☐ Air mixture used for states 1–3; products used for states 4–6.

☐ Combustion mass and elemental balances close.

☐ Diffuser, combustor and nozzle energy balances close.

☐ Compressor/turbine shaft balance closes under the specified shaft model.

☐ Isentropic reference-state entropy conditions are satisfied.

☐ State table and key outputs are printed cleanly.

☐ No hard-coded intermediate results remain.

☐ Report follows the current Canvas Word template.

☐ Submitted package runs from a fresh folder.

☐ Current 2026 naming, group-size and deadline rules checked on Canvas.

One-line project logic Physics → assumptions → mass/energy/entropy balances → NASA properties → solve state → check physics/residuals → pass state to next control volume.

End of working plan. Keep this file updated when the team confirms the missing official assumptions.